| Subdivision (Stage) / Worksite: |  |
| --- | --- |
| Date of Inspection : |  |
| Location: |  |

| Category | Checklist Items | Reference | Yes | No | N/A | MSC – HP (add initial next to box) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safety & Environment | All Employees Inducted and signed onto SWMS. All environmental controls implemented as per the approved EMP. |  | □ | □ |  |  |  |
| Setting Out | Check type of kerb from design drawing. Surveyor set out. Line to be within 10mm of the design. HP - Council will review and confirm the set out prior to commencement. |  | □ | □ | □ | □ |  |
| Bedding | Bedding material to be Class 3 Crushed Rock. Where not over pavement layers, minimum 75mm compacted thickness. Kerb bedding to extend 150mm past B.O.K. | IDM SD110 | □ | □ | □ |  |  |
| Hold Point | Proof Roll Kerb Pad Visual Inspection if insufficient width for watercart |  | □ | □ | □ | □ |  |
| Concrete Supply | Concrete used in kerb extrusion machines shall be: - Traffic Routes 320kg/m3 - Local Streets 280kg/m3 In-situ kerb with internal vibration shall be: - Traffic Routes – 32 MPa standard strength grade - Local Streets – 25MPa standard strength grade | VR 703.05 | □ | □ | □ |  |  |
| Pour Kerb | Concrete shall not be placed when the air temperature is less than 5 degrees C and above 35 degrees C. When necessary Concrete shall be cured either by water curing, wet hessian, polyethylene sheeting which is adequately sealed, curing compound or a combination of these. Freshly finished exposed concrete surfaces shall be effectively protected from rain or damage from other sources, until hard set has occurred. | VR 610.17 VR 703.10 | □ | □ | □ |  |  |
| Joints | Transverse Joints shall be constructed at regular intervals not exceeding 2.5m. | IDM SD105 VR 703.26 | □ | □ | □ |  |  |
| Existing Concrete | When constructing adjacent to existing concrete, the existing footpath or kerb shall be pinned using R10x300mm dowel bars at 600mm centres or min. 2 x dowels for kerb sections. |  | □ | □ | □ |  |  |
| Concrete Finish | Concrete to be smooth trowelled finished on tray and kerb. | IDM SD105 | □ | □ | □ |  |  |
| Post Pour Inspection | The concrete shall have no surface cracks at any stage after construction of width greater than 0.2 mm. Where such cracks exist, they shall be identified as a non-conformance. | VR 703.30 | □ | □ | □ |  |  |
| As-Cons | Survey pickup for as-constructed drawings in AutoCAD format. | IDM 7.6 |  |  |  |  |  |
| Consent to Proceed to Next Stage? |  |  | □ | □ |  | □ |  |

Additional Comments:

| Contractor/Superintendent Name: | Signature: | Date: |
| --- | --- | --- |
| MSC Construction Supervisor Name: | Signature: | Date: |